Requirements:

Main Overarching Goal (R0)

- The robot shall not fall over after initiated in the upright position.

- The robot is not required to pick itself up.

What does not falling over mean? (R1)

- The robot shall maintain +/- 5 degrees off of vertical for 15 mins.

Justification:

Falling over means the robot reaches a  value that is not recoverable, which is when the gravitational torque exceeds the motor torque capable.

τmotor=mghsinθ

h is the distance from the center of the wheel to the COM of the pendulum.

θ=sin-1(τmotormgh)

Assume: m = 2 kg, h = 0.12 m, τmotor = 1.765 Nm

θ=48.6°

This is the theta that the robot could not recover from.

To further narrow this down, I also need this robot to be able to handle having an object on top. So I need to figure out what  causes the object to fall off, which in other words is when gravity > friction.

Ffriciton=μmgcosθ

Fslide=mgsinθ

θ= tan-1μ

I only have PLA to make this robot chassis which has a  of 0.3 to 0.7. So, taking 0.3:

θ= 16.69°

So the 5° requirement provides a 10° safety factor from the theta that causes the object to slide off.

The 15 mins criteria is a bit made up but in an effort to make this a pure justification and learn a bit here goes. I will be using 18650 batteries (2 for Arduino and 4 for motors). They have a capacity of 3.5Ah each. So total Id have about 3.5Ah for the Arduino and since the cells will be wired in series to provide 12V for the motors it is 3.5Ah for the motors. Assume a worst case of 7A current draw from the motors (based on stall current for the motors I already bought) at steady state and 0.5A current draw from the Arduino. That would mean I can last for about 0.5 hours in a worst-case situation. Considering I am not good at this stuff, 15 mins is well within this constraint.

Recover from any poke force (R2)

- The robot shall be able to recover from any poke force applied to the top of robot.

- The robot shall be able to return to +/- 5 degrees from vertical in 1 second from a poke force.

Justification:

The +/- 5 degrees has been justified above.

The 1 second

- Impulse magnitude shall be XX N.

Justification:

The force applied by a finger poke should not generate a torque that exceeds the max torque that the motor can provide. This is not something that can be controlled but rather it’s to keep expectations realistic, that the robot should not be expected to recover from a poke that is too strong.

τpoke+τgravity=τmotor, max

Fl+ mghsinθ= τmotor, max

F should be less than τmotor, max-mghθl

Linearized sin about the vertical.

Recover from object placed on top (R2)

- The robot shall be able to recover in 1 second from an object placed on top.

Justification: See R2.1 from Recover from any poke force section.

- The payload mass shall be up to 0.5 kg.

Justification: I’d like to be able to balance a glass of water or something high stakes to see how it performs.

- With payload applied, the robot shall satisfy R1.1.

- The robot shall not need to handle poke forces when an object is placed on top.

Timing (R3)

- The robot shall measure its orientation and update motor commands at a fixed rate.

- The control update rate shall be XXX Hz.

Compute (R4)

- The hardware shall be able to complete all computations in one control period.

- Worst case execution time of the balance control loop shall be less than 1 ms.

- Balance control shall not be blocked by non-control tasks.

Future Vision System (R6)

- Robot shall be designed to allow for future upgrade involving full vision system

- Vision system shall allow robot to follow 1m behind person.

- Vision system shall communicate adjustments necessary to maintain 1m distance to control system.

- Vision system behaviors shall not interfere with balance control.

- Vision processing shall not execute within the balance control loop.

- Vision systems shall send updates every XXs.